- Guide to making an intel e1000 driver

- Setup:

- Download virtualBox and setup Ubuntu

- Go to vm settings -> network

- Set e1000 intel card

- Select attached to “Bridged Adaptor”

- Promiscuous mode: Allow All

- Connect to your host NIC

- There are 3 major components for NIC that you have to take care of other than the standard linux driver initialization:

- PCIE

- there are 3 ways to setup:

    - fully custom

- reading writing config header

    - unmanaged linux functions

- using pci_*

    - managed linux function (the one i used)

- using pcim_*

- this basically handles unloading/freeing automatically unlike others

status = pcim_enable_device(pci_dev);

pci_set_master(pci_dev);

status = pcim_iomap_regions(pci_dev, BIT(0), KBUILD_MODNAME);

ptr_bar0 = pcim_iomap_table(pci_dev)[0];

Together, these lines:

- Enable the PCI device.

- Allow it to do DMA.

- Reserve and map its BAR0 (register space).

- Get a kernel-accessible pointer to those registers.

pci_read_config_
byte: 1 byte
word: 2byte
dword: 4bytes
qword: 8bytes

- ioread32()//newer  =  writel()//older

- NET_DEV

- define

- allocate

- setup

- register

- link with pci obj

struct net_device *netdev;

netdev = alloc_netdev(sizeof(struct m_adapter), "mustafa%d", NET_NAME_UNKNOWN, m_setup);

// m_setup()-------------

ether_setup(dev); // sets up Ethernet-like device

dev->netdev_ops = &netdev_ops;

eth_hw_addr_random(dev); // assign random MAC

// ---------------------------

status = register_netdev(netdev);

pci_set_drvdata(pci_dev, netdev);

- Adapter

- Adapter struct is mainly made after reading docs to encapsulate the hardware complexities of the device except for the 3 fixed components:
1. pci_dev	2. net_dev	3. ptr_bar0 (hw_addr)

struct m_adapter {

struct pci_dev *pci_dev;

struct net_device *netdev;

void __iomem *hw_addr;

struct tx_desc *tx_ring;

dma_addr_t tx_ring_dma;

struct sk_buff *tx_skbs[NUM_TX_DESC];

u16 tx_next;

u16 tx_dirty;

struct rx_desc *rx_ring;

dma_addr_t rx_ring_dma;

struct sk_buff *rx_skbs[NUM_RX_DESC];

u16 rx_next;

int irq;

};

Descriptors:

struct tx_desc {

u64 buffer_addr;

u16 length;

u8 cso; // Checksum offset

u8 cmd; // Command flags

u8 status; // Status field

u8 css; // Checksum start

u16 special;

};

struct rx_desc {

u64 buffer_addr;

u16 length;

u16 checksum;

u8 status;

u8 errors;

u16 special;

};

format of descriptor on page 50 of sw guide

- rx/tx rings:

- "Ring" = array of descriptors.

- Each descriptor points to a packet buffer in memory.

- Hardware walks the ring and updates descriptor status.

- Software (driver) also walks the ring.

- rx_ring_dma:

- The DMA (bus) address of the RX descriptor ring.

- This is the device-visible address.

- NIC uses this address to read/write descriptors.

- Allocated using dma_alloc_coherent() usually.

- Driver writes this address into a NIC register.

- skb

- The Linux kernel’s main packet buffer structure.

- Abstracts packet + metadata.

- Contains pointers to actual data, protocol info, headroom, tailroom, etc.

- Used by networking stack, not hardware.

- Full driver flow:

- Init:

- insmod driver.ko

- ↓

- m_init() →  pci_register_driver()

- ↓

- PCI subsystem matches device

- ↓

- m_probe() →  register_netdev()

- ↓

- (ip link set ethX up)

- ↓

- m_open() →  mac_setup() →  tx_setup() →  rx_setup() → enable interrupts

netif_carrier_on(dev); // ip link set up from drivers perspective

- NIC will drop any unicast packets unless you program the MAC address into the NIC's RAL/RAH registers. (only broadcast would be seen)

static void m_setup_mac(struct m_adapter *adapter) {

void __iomem *hw = adapter->hw_addr;

u8 *mac_addr = adapter->netdev->dev_addr;

u32 ral, rah;

// Set Receive Address Low (RAL)

ral = (mac_addr[0] << 0) | (mac_addr[1] << 8) | (mac_addr[2] << 16) | (mac_addr[3] << 24);

writel(ral, hw + E1000_REG_RAL);

// Set Receive Address High (RAH) with Address Valid bit

rah = (mac_addr[4] << 0) | (mac_addr[5] << 8) | (1 << 31); // AV bit

writel(rah, hw + E1000_REG_RAH);

}

- TX:

- Linux → m_tx() → fill TX descriptor → write NIC doorbell

- |

- v

- NIC DMA reads skb data

netif_start_queue(dev);

- start taking packets from Linux network stack

- TCTL => for transmittion enable and init tx config

- TIPG => to control inter packet gap timer

- RX:

- NIC receives_packet

- |

- v

- updates RX descriptor status

- |

- v

- triggers interrupt → m_interrupt() → m_tx/rx_cleanup()

- |

- v

- driver allocates skb + copies data

- |

- v

- hands skb to network stack (netif_rx / NAPI)

netif_rx(skb);

- driver hands of data to Linux network stack

- my m_interrupt func calls m_rx_cleanup()

- Walk through RX descriptors in your ring.

- For each descriptor that hardware has filled (DD bit set):

- Unmap DMA.

- Build an skb.

- Pass packet to kernel networking stack.

- Refill that descriptor with a fresh buffer.

- Move the RX tail pointer (RDT) so NIC can use the new buffers.

- This is how packets move from NIC ➝ driver ➝ kernel.

while (processed < 16): limits work per interrupt (interrupt based polling not napi). added an if check in case packets end before 16

skb = netdev_alloc_skb(dev, 1536);

allocate skb

RCTL => for recieve enable and init rx config

- 1536 bytes:

| Component | Size |
| --- | --- |
| Max Ethernet payload | 1500 bytes |
| Ethernet header (dest+src+type) | 14 bytes |
| VLAN tag (optional) | 4 bytes |
| CRC/FCS | 4 bytes |
| Driver alignment/padding | ~8–16 bytes |

| Stage | CPU view | NIC view |
| --- | --- | --- |
| Before dma_map | virtual addr A | no access |
| After dma_map | virtual addr A | DMA addr dma |
| After dma_unmap | virtual addr A | access denied/invalid |

- Interrupts:

| Register | Purpose |
| --- | --- |
| ICR | Shows WHICH interrupt happened (status). Read-only. |
| ICS | software-only register that lets the driver simulate interrupts for degbugging |
| IMS | Enables interrupts (mask set). Write-only. |
| IMC | Disables interrupts (mask clear). |

- For the minimal driver i enabled only 3 interrupts:

- E1000_ICR_TXDW   → Transmit Descriptor Written

- E1000_ICR_RXT0   → Receive Timer (packet received)

- E1000_ICR_LSC    → Link Status Change

- Interrupt flow:

- Driver writes IMS to enable interrupts. e.g., TXDW, RXT0, LSC

- Hardware receives an event → sets a bit in ICR

- Hardware interrupt line fires

- Driver interrupt handler reads ICR

- Driver clears ICR bits by reading the register

status = pci_alloc_irq_vectors(pci_dev, 1, 1, PCI_IRQ_LEGACY | PCI_IRQ_MSI); if (status < 0) {goto irq_fail_1;}

adapter->irq = pci_irq_vector(pci_dev, 0);

status = request_irq(adapter->irq, m_interrupt, IRQF_SHARED, netdev->name, netdev); if (status) {goto irq_fail_2;}

- kernal automatically chooses between legacy and msi

- why 0? we requested only one interrupt in prev line

- 3rd line registers interrupt handler function with the kernal

| Type | Bus support | Max vectors | Trigger | Name |
| --- | --- | --- | --- | --- |
| Legacy (INTA-INTD) | PCI,PIC-X | 1-4 per device | Level | - Pysical pins - Shared - slower |
| MSI (Message signaled interrup) | PCI-X, PCIe | 1-32 (power of 2) | Edge | - Memeory write to speicial address - no sharing - better performance |
| MSI-X | PCI-X, PCIe | till 2048 | Edge | - data per vector (independant address) - ideal for multi queue driver - most felxible |
| INTx emulation | PCIe | 1 | Emulated | - uses special pcie messages (assert/dessert) for emulation |

Summary:

Compulsory Responsibilities (Your Driver MUST Handle)

- Hardware Initialization/Shutdown:

- PCI resource mapping (pcim_enable_device, pcim_iomap_regions)

- Descriptor ring setup (TX/RX DMA memory allocation)

- Hardware register configuration (TCTL/RCTL, descriptor pointers)

- Interrupt setup/cleanup (request_irq, free_irq)

- Core Netdev Operations:

- .ndo_open: Enable hardware, start queues (netif_start_queue), enable interrupts

- .ndo_stop: Disable hardware, stop queues (netif_stop_queue), disable interrupts

- .ndo_start_xmit: Transmit packets (DMA mapping, descriptor management)

- Link state management: Report carrier status (netif_carrier_on/off)

- Resource Management:

- DMA mapping/unmapping for TX/RX buffers

- SKB lifecycle management (alloc/free in RX path, free after TX completion)

- Proper cleanup of all resources on removal (rings, IRQs, netdev)

- Interrupt Handling:

- Acknowledge hardware interrupts (read ICR register)

- Process TX completions (free SKBs, update ring pointers)

- Process RX packets (pass to stack via netif_rx)

- Handle critical errors (no buffers, DMA errors)

- Basic Statistics:

- Update dev->stats.tx_packets/bytes and rx_packets/bytes

- Track errors (drops, DMA failures)

Handled by Linux Network Stack (You DON'T Implement)

- Protocol processing: IP/TCP/UDP checksums (unless offloaded)

- Packet scheduling: QoS, traffic shaping

- Socket API handling: send()/recv() syscalls

- Routing decisions: Forwarding tables

- Buffer management: SKB allocation (though you do need to allocate RX buffers)

- Offload processing: TSO/GSO/GRO (unless explicitly supported)

Handled by NIC Hardware (You Configure but Don't Emulate)

- DMA engine: Moving packet data between host memory and NIC

- Packet framing: Adding/removing Ethernet FCS (if enabled)

- Interrupt generation: TX completion, packet arrival, link changes

- Hardware filtering: MAC address matching (if enabled)

- Flow control: Pause frames (if configured)

Optional but Recommended Features

| NAPI | Critical for high-speed RX performance |
| --- | --- |
| ethtool support | Diagnostics (ethtool -S eth0) |
| Link state polling | Real-time carrier detection |
| Multicast support | Required for IPv6, mDNS, etc. |
| Watchdog timer | Recover from TX stalls |
| Hardware offloads | Checksum/TSO (for performance) |

PCIe → main data interface between the NIC and the host (for sending/receiving packets).

SMBus → sideband management interface (for power management, wake-on-LAN, configuration, etc.), not for data transfer.

All inits for a network driver:

- Linux module init

- PCIe init

- Net_dev init

- RX init

- TX init

- irq init

- mac init